🔬 Why Graphite is a Good Conductor of Electricity

- Atomic Structure of Graphite

  - Graphite is made of carbon atoms arranged in flat layers (called graphene sheets).

  - Each carbon atom is bonded to only 3 other carbon atoms in a hexagonal pattern.

- Delocalized Electron

  - Carbon has 4 valence electrons. In graphite:

    - 3 electrons form strong covalent bonds with neighboring atoms.

    - The 4th electron is free — it does not bond.

  - This free electron is delocalized, meaning it can move freely across the layer.

- Electron Mobility = Conductivity

  - These mobile electrons act like a "sea of charge" — similar to metals.

  - They can carry electric current when a voltage is applied.

- Layer Structure Helps

  - The layers are held together by weak forces (van der Waals), so electrons can move easily within layers but not between them.

  - That's why graphite conducts in the plane of the layers, but not vertically.

Why do electrons have a negative charge?

Electrons are assigned a negative charge by convention, primarily because Benjamin Franklin's early experiments led to the assumption that the charge acquired by a glass rod rubbed with silk was "positive," and the opposite charge was then termed "negative".

Here's a more detailed explanation:

- Historical Context:

Benjamin Franklin, in his experiments with electricity, established the concept of positive and negative charges.

    Franklin's Choice:

He arbitrarily designated the charge acquired by a glass rod rubbed with silk as "positive," and the opposite charge as "negative".

    Electron Discovery:

Later, when electrons were discovered, they were found to be the carriers of the "negative" charge, and the positively charged particles were called protons.

    Convention:

This initial designation by Franklin, although arbitrary, has become the standard convention in physics and chemistry.

    Opposite Charges:

It is important to note that the charge of a proton and an electron are opposite, so that a neutral atom has the same number of protons and electrons, resulting in zero net charge

Total energy of the electron: Slide 32:

Newton's second law of motion:

At any instant of time, the net force on a body is equal to the body's acceleration multiplied by its mass or, equivalently, the rate at which the body's momentum is changing with time.

energy levels are quantized, meaning that a system can only exist at specific, discrete energy values, rather than a continuous range of values

🧠 Explanation of Each Row

1. Quantized orbits (n=1,2,…)(n = 1, 2, \ldots)(n=1,2,…) → Continuous energy bands

- Bohr Atom:
In the hydrogen atom, electrons can only occupy discrete orbits (or energy levels), labeled by an integer quantum number nnn. Each level corresponds to a specific energy WnW_nWn​.

- Solid Material:
In a solid, atoms are packed closely together. Their overlapping orbitals cause these discrete levels to broaden into continuous energy bands, because electrons can occupy a range of closely spaced energy states.

2. Negative Wn​ = bound → Valence band (bound states)

- Bohr Atom:
The negative energy Wn<0 indicates that the electron is bound to the nucleus. The more negative it is, the more tightly the electron is held.

- Solid Material:
Electrons in the valence band are also in bound states — they are still associated with specific atoms and cannot move freely through the material without additional energy.

3. Ionization limit W=0 → Bottom of conduction band

- Bohr Atom:
When W=0W, the electron is no longer bound to the atom — it has enough energy to escape (i.e., it’s ionized).

- Solid Material:
The bottom of the conduction band is the lowest energy an electron can have while being "free" to move through the material. It's like the “ionized” state in an atom, but in a crystal lattice.

🧩 Why this table matters:

It shows that:

- Atomic discrete energy levels evolve into energy bands in solids.

- The bound vs. free nature of electrons in atoms carries over to the valence vs. conduction band picture.

- Bohr’s quantum model forms a foundation for understanding electrical behavior in materials — especially in semiconductors and insulators.

This slide explains how energy bands form in solids and the role of the quantum number nnn in transitioning from discrete atomic levels (Bohr model) to continuous bands in solid-state materials. Here's a breakdown of the key points:

🔹 Valence Band

- Formed from the outermost electron levels of atoms (e.g., quantum levels where n=2,3n = 2, 3n=2,3).

- These electrons are bound and not free to move easily.

- In solids, these orbitals overlap, and the discrete energy levels merge into a band.

🔹 Conduction Band

- Formed from higher energy states that are unoccupied in a neutral atom.

- Electrons in this band are free or nearly free, contributing to electrical conduction.

🔹 Band Gap (EgE_gEg​)

- The energy difference between the conduction band and valence band.

- A key factor in determining whether a material is a conductor, semiconductor, or insulator:

  - Small EgE_gEg​ → semiconductor

  - Large EgE_gEg​ → insulator

  - No EgE_gEg​ (overlap) → conductor

🔹 Energy Bands & Role of Quantum Number nnn

- In the Bohr model, nnn defines discrete energy levels (orbits).

- In solids, due to atomic proximity, orbitals overlap, and these discrete levels form bands.

- These energy bands are separated by band gaps, and this structure governs electrical behavior.

-------------------------------------------------------------

Acceptor and donor:

If a small percentage of trivalent or pentavalent atoms are added to intrinsic silicon or germanium, a doped, impure, or extrinsic semiconductor is formed.

-------------------------------------------------------------

Lecture II: 16-04-2025

Mass action law: Derivation:

Analogy to understand: NC​ as the number of available hotel rooms (states) in the conduction band. Electrons (guests) can move in if energy (money) allows — but the rooms must exist first!

Charge neutrality equation

Slide 53: Conductivity and career mobility: 
?????? Why

Relevance of this equation:

| Symbol | Meaning |
| --- | --- |
| nin_ini​ | Intrinsic carrier concentration (number of free electrons or holes in an undoped semiconductor) |
| A0 | A material-dependent constant that includes the effective density of states in conduction and valence bands |
| T | Absolute temperature (in Kelvin) |
| T3 | Shows that the number of available energy states increases with TTT |
| EG0 | Bandgap energy at 0 K (energy needed to excite an electron across the gap) |
| K | Boltzmann constant ≈1.38×10−23 J/K\approx 1.38 \times 10^{-23} \, \text{J/K}≈1.38×10−23J/K |
|  | Exponential part that shows how hard it is to excite electrons across the bandgap (falls with larger bandgap or lower temperature) |

Hall effect:

Case I:

No magnetic field:

Magnetic field is applied:

The magnetic field is into the metal and then impact the flow of direction of electroncs.

So now the electrons and holes are forming like a plate.

Fe is electric filed and Fm is magnetic field.

This above will form plates in equilibrium. The difference in potential between the two plates is the hall voltage.

This is how we calculate the hall voltage.

Relevance of hall coefficient: